TD 5G NR

Exercise 1: 5G NR features

What is, in your opinion, the main factors that make that 5G NR provides a better throughput than LTE and LTE-A?

- Larger bandwidth (new available frequencies)

- Larger subcarriers : 30khz, 60khz and 120khz

Exercise 2: Throughput computation

Orange France uses a frequency band of 90MHz in the 3.5Gz spectrum band for the deployment of its 5G NR network. The frequency band is organized into 3000 subcarriers of 30KHz in FDD mode.

What is the maximum theoretical downlink data rate that can be provided by Orange under good radio conditions allowing the use of 256-QAM modulation (normal cp mode)?

1 ts = 0.25ms = 7 symbols = 7*8 bits = 56 bits

Datarate of 1 sc = 56bits / 0.25ms = 224 kbps

Datarate of a downlink is = 224 kbps * 3000 sc = 672Mbps

Exercise 3: Antenna configuration

5G NR gives many options that allow a better adaptation of the network for the corresponding environment. Using arrows propose for each class of environment, the adequate frequency band, subcarrier wideband and cyclical mode.

| Environment | Frequencies | Subcarrier size | Cyclical mode |
| --- | --- | --- | --- |
| Paris-centre + dense network Montbéliard + dense network Montbéliard + low dense net. A36 Belfort-Montbéliard Saint-Jean-Pied-de-Port | 3.5GHz 24GHZ 3.5MHz 700MHz 700MHz | 15KHz 30KHz 60KHz 60Khz/120KHz 30KHz | Normal Normal Normal/Extended Extended Extended |

Exercice 4.  5G NR Throughput according to the CQI

Hereafter the correspondence table between the SINR observed by the UE and the CQI.

- Calculate the provided 5G NR downlink throughput if the available BW is of 70 MHz, the frequency band is of 4.6GHz, the cyclic prefix mode is extended, the transmission power of the antenna is of 43dBm, the distance of the UE from the antenna is of 1400m. The subcarrier size is of 15kHz. The computing should consider the nose+interference (= -100dBm), and the free space propagation model (loss = 32,45 dB + 20*log [frequency (MHz)] + 20*log [distance (km)]).

Received signal = 43dBm -Loss = 43dBm - 32,45 dB - 20*log [4600] - 20*log[1.4])

Received  signal = 11db - 3.66*20 - 2.92 =  -65.12dBm

SINR = +-65.12—100= 35db

Modulation scheme is 256QAM

4660 subcarriers

1 ts = 0.5ms = 6 symbols = 6*8 bits = 48bits

1 subcarrier, datarate = 96 kbps

Datarate = 4660 * 96 kbps = 447Mbps

Utile datarate = Datarate * 948/1024 = 413 mbps

- What is the useful download throughput?